Care and Maintenance Sheet

March 2020

Stone: Bluestone

NB: These are general recommendations only, for technical advice please don’t hesitation to call iRock Finishes - 1300 131 271

Bluestone Recommended Care and Maintenance

Daily/Weekly:

- Mop up spills as soon as possible, using a pH neutral cleaner.

- For general cleaning, mop over all surfaces using a pH neutral cleaner at a ratio 1:100 with warm or cold water.

- For problem spots:

  - use a doodle bug fitted with a blue pad and a pH neutral cleaner on the problem area

  - mop the area with a damp mop.

- Please have us advise on heavy spills (oils, acidic, salts, etc.), although most can be corrected with similar methods as above, using appropriate cleaners and/or diamond polishing pads.

6-12 Months:

- Slowly pass over entire surface with an auto scrubber and/or pressure washer with a pH Neutral cleaner. Remove excess water with a wet vacuum and mop according to instructions above.

- For edges:

  - use a doodle bug fitted with a blue pad and pH neutral cleaner

  - mop over edges with a damp mop.

3-5 Years:

- Consider resealing your bluestone.

.